Serato SSL Midi Commands for

American Audio Radius 1000

Using the Radius 1000 with Serato SSL requires an interface Like the SL1, or SL3, to use the software. The Radius 1000 gives you the advantage of using the Radius 100 Midi feature with Serato V1.8 or Later.

Connect your setup according to manufactures instructions. Connect the USB(s) from the Radius (or multiple Radius players) to your computer. If your computer needs a USB Hub to support more than 3 devices we recommend you use a high speed model.

Scratching in MIDI

Enter the Set up menu on the Radius by holding down the Folder Knob. Select B. MIDI set up, Activate the Dual Mode on, this allow you to use the MIDI mode while having some controls of your Tone/Time code CD’s. This function allows you to have normal CD player controls of the Pitch fader, Pitch on/ off button, Pitch % values, And Jog Wheel Control. * to Switch  Jog wheel modes simply deactivate MIDI mode temp. set to Normal for pitch bend, and Scratch for Scratching.

If you are using two Radius 1000s we recommend that you set left Deck to MIDI Ch1 and the Right Deck to MIDI Ch. 2. If you only wish to use one Radius you can still control both decks with one control. If this is the case set it to midi CH1/2 and use the P. button to toggle between Ch1 or 2.* This will require you to use the input reverse input on SSL.

It is also necessary to change your MIDI CC type on the encoders to Absolute with SSL. If you choose Dual Mode Enable Pitch, Disable pitch lock and set desired Jog mode Scratch or Normal.

If you wish to Scratch you must have a control CD and the unit in Dual mode or toggle the MIDI on/off. SSL only scratches in Absolute mode and Internal.

| Midi Control | Software Function |
| --- | --- |
| Folder Knob Turn | Scroll up/ Down Thru Folder Browser and Tracks |
| Folder Knob Push | Switch between Folders and Tracks List. |
| Track Knob Push | Load Selected Track |
| Adv. Button | Add Selected Song to Prepare List |
| Tap+  Fast Tack | UP |
| Tap+  Fast Tack | Left |
| Tap+ Search | Down |
| Tap+  Search | Right |
|  Fast Tack | Next song in List |
|  Fast Tack | Previous song in List/ return to beginning of song |
|  Search | Rewind |
|   Search | FFWD. |
| Filter | Open / Close Browse Mode |
| Echo | Review |
| Trans | Prepare List |
| Skid | Import |
| IN | Set Loop in |
| Out | Set Loop out |
| Reloop | Disable Loop |
| Tap+ Filter | 1/8 Note Loop |
| Tap+ Echo | ¼ Note Loop |
| Tap+ Trans | ½ Note Loop |
| Tap+ Skid | 1 Note Loop |
| Tap+ Hold | 2 Note Loop |
| Tap+ Phase | 4 Note Loop |
| Tap+ Flang | 8 Note Loop |
| Tap+ Pan | 16 Note Loop |
| Tap+Rev | 32 Note Loop |
|  Beat | Moves Beat View Left |
|  Beat | Moves Beat View Right |
| SGL/ CTN | REPEAT on/off |
| Cue | Set Temporary Cue |
| Tap+ Cue | Temp Cue |
| CUE #1 | Play 1st Cue |
| CUE #2 | Play 2nd Cue |
| CUE #3 | Play 3rd Cue |
| P. *** | Play 4th Cue |
| S | Play 5th Cue |
| Tap+ CUE #1 | Set/overwrite 1st Cue |
| Tap+ CUE #2 | Set/overwrite 2nd Cue |
| Tap+ CUE #3 | Set/overwrite 3rd Cue |
| Tap+ P. *** | Set/overwrite 4th Cue |
| Tap+ S | Set/overwrite 5th Cue |
| Parameter Time Turn | Deck Gain |
| Parameter Ratio Turn(0-127) | Brake (Min – Max time) |
| Eject | Dump song from Deck |
| Pitch Fader+++ Dual mode off | Pitch Fader INT mode only+++ |
| Lock Button | Key Lock |
| Play | Play/Pause Int Mode only |
| Tap + Play | Reverse Play/Pause int mode only |
| Modes |  |
| A. cue Scratch | Abs. |
| Scratch | Rel. |
| Normal | Int. |